Scotsman Commercial Ice Systems

Technical Update

December 2010

Scotsman’s MDT5 series of Nugget ice maker-dispensers was recently changed to include a fixed ice spout. This new spout, seen in Service Bulletin PS-8-2010 on this website, is available for retrofit on prior units.

Prodigy Cuber Controller Clean Mode Timings

Controller Revs 1, 2 and 3:

- Clean Phase 1: The water pump is on all the time. Purges and refills once by sump empty and sump full signals.  Once refilled it will then just circulate the water (no additional purges).

- Clean Phase 2: The water pump is on all the time. Purge valve opens every 20 seconds for 30 seconds. Inlet water valve opens to refill ONLY if the sump empty light switches on.

Controller Revs 4 through 7:

- Clean Phase 1: The water pump is on all the time. The purge valve opens for 2 minutes and then closes to stop draining water. The inlet water solenoid valve then opens to refill the reservoir after the two minutes but only if the sump empty light is ON within those two minutes. If the sump empty light does NOT come on during the 2 minute purge time, the sump will not refill, even if the sump empty light switches on later.

- Clean Phase 2: The water pump is on all the time. The purge valve opens every 20 seconds for 30 seconds, then the inlet water valve opens and refills the reservoir UNLESS the Sump FULL light is on.

Note: If the water level sensor is not clean, and indicates sump full when it isn’t, then the refill processes listed above will not occur.